Modeling_2026 Project Recreation Guide

Updated with Model Quality Recommendations

Version 2.0 - January 26, 2026

# ⚠️ CRITICAL UPDATE: Model Quality vs. Instruction Count

KEY INSIGHT: Having all instructions does NOT make a model "best" for grey-box queueing methodology.

What Actually Matters for Performance Modeling:

- Accurate timing for instructions that dominate workloads (typically 10-20 instructions account for 80%+ of execution)

- Correct architectural bottleneck modeling (prefetch queues, memory wait states, cache effects)

- Validation against real hardware measurements

## The Problem with Exhaustive Instruction Lists

| Approach | Example | Issue |
| --- | --- | --- |
| Exhaustive | 230 instructions | Includes 49 MOV variants that all have identical 5 T-state timing |
| Category-based | ~20 categories | Groups MOV r,r as one category with 5 T-states - captures behavior efficiently |

## Recommended Approach: Category-Based Timing

For grey-box queueing models, instruction categories with weighted averages are more useful than exhaustive instruction lists:

# Preferred model structure
TIMING_CATEGORIES = {
    "register_alu": {"cycles": 5, "weight": 0.35},
    "memory_load": {"cycles": 8, "weight": 0.20},
    "memory_store": {"cycles": 7, "weight": 0.15},
    "branch_taken": {"cycles": 11, "weight": 0.10},
    "branch_not_taken": {"cycles": 5, "weight": 0.10},
    "immediate": {"cycles": 8, "weight": 0.10},
}

## Quality Comparison: Good vs. Bloated Models

| Model | Instructions | Quality | Assessment |
| --- | --- | --- | --- |
| 4004/6502 | 46-56 | IDEAL | Category-based, captures workload behavior efficiently |
| Intel 8008 v2 | 230 | BLOATED | 49 MOV variants that all have identical timing |
| Intel 4040 v2 | 153 | BLOATED | 16 variants each of INC, ADD, SUB, LD, XCH |
| Intel 80186 v2 | 156 | GOOD | Covers actual instruction timing differences vs 8086 |
| AMD 2901 v2 | 8 ALU ops | CORRECT | Appropriate level for bit-slice (microcode defines instructions) |

# Refined Methodology for Maximum Accuracy

## Model Structure Guidelines

DO:

- Use instruction CATEGORIES (register ALU, memory load, branch, etc.)

- Include weighted averages based on typical workload profiles

- Model architectural bottlenecks (prefetch queue, bus width, cache)

- Validate against documented benchmarks or emulator measurements

- Focus on the 10-20 instruction types that dominate execution

DON'T:

- List every register variant separately (MOV A,B / MOV A,C / MOV A,D...)

- Confuse instruction count with model quality

- Create redundant entries that all have the same timing

- Prioritize completeness over usefulness for analysis

## Ideal Model Components

| Component | Content |
| --- | --- |
| Instruction Categories | 5-15 timing categories with cycle counts and workload weights |
| Architectural Parameters | Clock, bus width, prefetch queue size, cache specs (if any) |
| Bottleneck Model | M/M/1 queues for fetch, decode, execute, memory stages |
| Workload Profiles | 3-5 profiles (typical, compute, memory, control, I/O) |
| Validation Suite | Test cases with expected IPS/MIPS values from datasheets |

## Example: Ideal 8008 Model Structure

# GOOD: Category-based model
INSTRUCTION_CATEGORIES_8008 = {
    "mov_reg_reg": {
        "description": "Register-to-register move (MOV r1,r2)",
        "t_states": 5,
        "clock_cycles": 10,  # 5 T-states × 2 clocks/T-state
        "typical_weight": 0.25
    },
    "mov_reg_mem": {
        "description": "Memory load/store (MOV r,M / MOV M,r)", 
        "t_states": 8,
        "clock_cycles": 16,
        "typical_weight": 0.15
    },
    "alu_register": {
        "description": "ALU with register (ADD/SUB/AND/OR/XOR r)",
        "t_states": 5,
        "clock_cycles": 10,
        "typical_weight": 0.20
    },
    "immediate": {
        "description": "Immediate operations (MVI/ADI/etc)",
        "t_states": 8,
        "clock_cycles": 16,
        "typical_weight": 0.10
    },
    "jump_call": {
        "description": "Control transfer (JMP/CALL/RET)",
        "t_states": 11,  # Taken
        "clock_cycles": 22,
        "typical_weight": 0.15
    },
    "io": {
        "description": "I/O operations (IN/OUT)",
        "t_states": 8,
        "clock_cycles": 16,
        "typical_weight": 0.05
    }
}
# Total: 6 categories vs 230 individual instructions
# Same analytical power, much more useful for performance analysis

# Complete Processor Inventory

The project covers 63+ processor models from 1971-1985:

### Intel Family (18 models)

4004, 4040, 8008, 8080, 8085, 8086, 8088, 80186, 80188, 80286, 80386, 8048, 8051, 8748, 8751, iAPX 432, 80287, 80387

### Motorola Family (12 models)

6800, 6802, 6809, 68000, 68008, 68010, 68020, 6801, 6805, 68HC11, 68881, 68882

### MOS/WDC Family (4 models)

6502, 6510, 65C02, 65816

### Zilog Family (7 models)

Z80, Z80A, Z80B, Z8000, Z80000, Z8, Z180

### Other Manufacturers (22 models)

RCA 1802/1805, National SC/MP, Fairchild F8, AMD 2901/2903/Am29000, TI TMS9900/9995/TMS320, Signetics 2650, NS32016/32032, MIPS R2000, SPARC, ARM1, Transputer T414, WE 32000, Harris RTX2000, Novix NC4016

# Master Prompt for Project Recreation

Use this prompt to recreate the entire Modeling_2026 project with consistent, high-quality models.

## The Prompt

Create a grey-box queueing performance model for the [PROCESSOR NAME].
METHODOLOGY REQUIREMENTS:
1. Use INSTRUCTION CATEGORIES (not exhaustive instruction lists)
   - Group instructions by timing behavior (register ALU, memory, branch, etc.)
   - Include weighted averages for typical workloads
   - 5-15 categories is ideal; 230 instructions is NOT better
2. Model ARCHITECTURAL CHARACTERISTICS:
   - Fetch mechanism (sequential/prefetch queue/cache)
   - Bus width and memory timing
   - Pipeline stages (if any)
   - Key bottlenecks for this architecture
3. Include VALIDATION:
   - Expected IPS/MIPS range from datasheets
   - At least 3 test cases
   - Comparison against emulator or real hardware data
4. Create these files:
   - [processor]_model.py: Python implementation with category-based timing
   - [processor]_model.json: Configuration with workload profiles
   - README.md: Quick start guide
KEY TIMING SOURCES (in priority order):
1. Original manufacturer datasheets
2. Cycle-accurate emulators (MAME, VICE, etc.)
3. Published benchmarks (BYTE Magazine Sieve, etc.)
ACCURACY TARGETS:
- IPC prediction within 5% of measured values
- Instruction timing matches datasheet exactly
- Bottleneck identification correct for era
DO NOT create exhaustive instruction lists with register variants.
DO focus on category-based timing that captures workload behavior.

## Per-Processor Customization

Add these specifics based on processor era:

| Era | Additional Requirements |
| --- | --- |
| 1971-1976 (4-bit/early 8-bit) | Model machine cycle structure, BCD arithmetic timing, limited addressing modes |
| 1976-1980 (8-bit golden age) | Model prefetch effectiveness, page boundary penalties, interrupt latency |
| 1978-1983 (16-bit) | Model prefetch queue depth, EA calculation overhead, segment register effects |
| 1982-1985 (32-bit/cache) | Model cache hit/miss, MMU overhead, virtual memory effects, coprocessor wait states |
| 1985 (early RISC) | Model pipeline stages, load delay slots, branch delay slots, register windows |

Modeling_2026 Project Recreation Guide

Version 2.0 - Updated with Model Quality Recommendations

Grey-Box Performance Modeling Research

January 26, 2026