Lessons Learned: Grey-Box Queueing Models

for Pre-1986 CPU Performance Modeling

Martin Gallagher | January 2026 | Modeling_2026 Project

# Executive Summary

Over 63 processor models spanning 1971-1985 have been developed using grey-box queueing theory methodology. This document synthesizes the key lessons learned that enable consistent creation of accurate, interpretable performance models. The methodology achieves typical accuracy of <5% error after calibration while maintaining computational tractability and providing deep architectural insight.

# 1. Core Methodology That Works

## 1.1 The Grey-Box Philosophy

The most important insight is combining three types of knowledge:

- White-Box (Known from Architecture): Pipeline stages, clock frequency, bus width, register counts. These come directly from datasheets and technical manuals.

- Grey-Box (Measured from System): Cache miss rates, instruction mix profiles, memory access patterns. These require profiling or emulator instrumentation.

- Black-Box (Calibrated Iteratively): Unknown contention effects, unmodeled delays, effective prefetch rates. These are fitted to match measured IPC.

This approach yields better results than pure white-box (too many unknowns) or pure black-box (requires excessive data, loses interpretability).

## 1.2 M/M/1 Queueing Networks

Model each pipeline stage as an M/M/1 queue (Markovian arrivals, Markovian service, single server). For series networks (Jackson networks), key formulas are:

| Formula | Description |
| --- | --- |
| ρᵢ = λ × Sᵢ | Utilization: must be < 1.0 for stability |
| Lᵢ = ρᵢ / (1 - ρᵢ) | Queue length: average number in stage |
| Wᵢ = Sᵢ / (1 - ρᵢ) | Wait time: total time in stage |
| CPI = Σ(Wᵢ × f_clock) | Total cycles per instruction |
| IPC = 1 / CPI | Instructions per cycle |

Why This Works: Even though real instruction service times are not truly exponential, the M/M/1 approximation captures the essential queueing behavior. The calibration step compensates for the approximation error.

## 1.3 Calibration Protocol

The standard calibration workflow that consistently achieves <5% error:

- Run benchmark on real hardware or cycle-accurate emulator → collect measured IPC

- Profile workload → extract instruction mix (% ALU, MUL, DIV, MEM, etc.)

- Set known parameters from architecture documentation

- Binary search on arrival rate λ until |IPC_predicted - IPC_measured| < tolerance

- Validate bottleneck identification matches expected behavior

# 2. Architectural Lessons by Era

## 2.1 Simple Sequential Processors (1971-1976)

Examples: Intel 4004, 8008, 8080; MOS 6502; Motorola 6800

- Single-stage execution model works well. No overlap between fetch and execute means one M/M/1 queue often suffices.

- Weighted average service time captures instruction mix. S_exec = Σ(pᵢ × cycles_i) where pᵢ is probability of instruction type i.

- Memory access is the dominant factor. Most time spent fetching instructions and operands from slow RAM.

- The 6502 demonstrates that simplicity wins efficiency. 3,510 transistors achieved 5× IPC of the 8080's 4,500 transistors through clever design.

## 2.2 Prefetch Queue Processors (1976-1982)

Examples: Zilog Z80, Intel 8086/8088, Motorola 68000

- Prefetch effectiveness is the critical calibration parameter. Ranges from 60% (branch-heavy code) to 95% (sequential code).

- Model BIU and EU as separate queues. Bus Interface Unit fills prefetch queue while Execution Unit consumes it.

- Bus width creates real bottlenecks. The 8088's 8-bit bus made it 30-40% slower than the identical 8086 with 16-bit bus.

- Use finite-buffer M/M/1/K for prefetch queues. The 8086's 6-byte queue and 68000's 2-word queue are capacity-limited.

## 2.3 Cache and Virtual Memory Processors (1982-1986)

Examples: Intel 80286, 80386; Motorola 68020; MIPS R2000; ARM1

- Add dedicated MMU/TLB stage for protected mode processors. Address translation adds 1-5 cycles depending on TLB hit rate.

- Cache hit/miss modeling is essential. Service time = p_hit × T_hit + p_miss × T_miss creates dramatic performance swings.

- RISC processors need pipeline hazard modeling. Load delays, branch penalties, and structural hazards affect throughput.

- The 80386 marks the transition point. With on-chip cache and full 32-bit bus, memory bottleneck shifts to cache behavior.

# 3. Implementation Patterns That Work

## 3.1 Standard Directory Structure

Each processor model should have this consistent structure:

| File | Purpose |
| --- | --- |
| {cpu}_cpu_model.json | All parameters, instruction timings, architecture specs |
| {cpu}_cpu_model.py | Python implementation with calibration and analysis |
| {CPU}_QUEUEING_MODEL.md | Full technical documentation (50-70 pages) |
| QUICK_START.md | 5-minute getting started guide |
| README.md | Overview and navigation |

## 3.2 JSON Configuration Pattern

Structure the JSON configuration with these sections:

- architecture: Year, designer, bus widths, register descriptions

- pipeline: Stage names, base cycles, queue types, conditional flags

- instruction_timings: Cycle counts for each instruction type/addressing mode

- instruction_mix: Default workload profile (can be overridden)

- calibration: Tolerance, max iterations, parameter bounds

- validation: Known benchmarks with expected results

## 3.3 Python Implementation Pattern

Key classes and methods that should be present:

- @dataclass QueueMetrics: stage, service_time, arrival_rate, utilization, queue_length, wait_time

- @dataclass CalibrationResult: converged, iterations, error_percent, predicted_ipc, bottleneck

- compute_stage_metrics(λ): Returns list of QueueMetrics for all stages

- predict_ipc(λ): Returns (ipc, metrics) tuple

- calibrate(measured_ipc, tolerance): Binary search to match measured performance

- find_bottleneck(metrics): Returns stage with highest utilization

# 4. Validation That Builds Confidence

## 4.1 Multi-Benchmark Validation

Test each model against at least three distinct workload types:

| Benchmark Type | Characteristics | Expected Bottleneck |
| --- | --- | --- |
| Dhrystone | Integer-heavy, register operations | Execute stage |
| Memory Copy | Load/store heavy, sequential | Memory/BIU stage |
| Branch Heavy | Control flow, prefetch flushes | Prefetch effectiveness |
| Mixed | Realistic application profile | Varies by workload |

## 4.2 Data Sources for Validation

- Cycle-accurate emulators: MAME, Vice, Hatari, DOSBox-X, 86Box provide ground truth

- Original datasheets: Intel, Motorola, Zilog official documentation has instruction timing

- Historical benchmarks: BYTE Magazine, PC Magazine comparative reviews from the era

- Reference sites: WikiChip, CPU-World, Visual6502, pagetable.com historical archives

## 4.3 Accuracy Targets

| Metric | Target | Acceptable |
| --- | --- | --- |
| IPC prediction error | < 2% | < 5% |
| Calibration iterations | < 10 | < 20 |
| Bottleneck identification | Correct stage | Adjacent stage |
| Sensitivity direction | Correct sign | Correct magnitude |

# 5. Key Insights from 63 Models

## 5.1 Performance Evolution Patterns

- 1971-1978: Memory-bound era. IPC limited to 0.10-0.25 by slow RAM access. No overlap between fetch and execute.

- 1978-1982: Prefetch revolution. 8086's prefetch queue enabled 5.7× IPC improvement over 8080. Decoupling fetch from execute was transformative.

- 1982-1985: Cache hierarchy emergence. On-chip cache (80386) shifted bottleneck from memory to execution. IPC approached 0.90 for RISC designs.

- 1985: RISC inflection point. MIPS R2000 achieved 0.80 IPC vs 80386's 0.30, proving simpler ISAs enable higher performance.

## 5.2 Surprising Discoveries

- Z80 has same per-clock performance as 8080. Despite enhanced microarchitecture, the added complexity didn't improve IPC.

- Bus width matters more than clock speed. 8088 at 4.77MHz was slower than 8086 at 4.77MHz purely due to 8-bit vs 16-bit bus.

- The biggest single jump was 8080 → 8086. Prefetch queue introduction, not cache, caused the largest performance gain in this era.

- Transistor efficiency peaked with 6502. At 99.7 IPC per 1000 transistors, the 6502 remains the most efficient design in history.

## 5.3 Design Principles Validated

- Simplicity wins efficiency. Fewer transistors, cleaner design, better IPC (6502 vs 8080).

- Decouple fetch from execute. Prefetch queues hide memory latency effectively.

- Orthogonal ISAs enable optimization. 68000's regular encoding was loved by compilers, easier to optimize.

- Compatibility creates ecosystems. Z80's 8080 compatibility captured entire CP/M market despite lower IPC.

- The memory wall is eternal. Every generation fights the same battle: CPU speed vs memory latency.

# 6. Prompt for Modern CPU Models

Use this prompt to generate high-quality queueing models for CPUs from any era. Copy this entire section.

=== BEGIN PROMPT ===

Create a comprehensive grey-box queueing model for the [CPU NAME] processor. Follow my project's established methodology from Modeling_2026 (https://github.com/MartinGallagher-code/Modeling_2026).

REQUIRED DELIVERABLES:

1. {cpu}_cpu_model.json - Complete configuration with:

- Architecture section (year, designer, bus widths, registers)

- Pipeline section (stages with base cycles and queue types)

- Instruction timings (all addressing modes with cycle counts)

- Default instruction mix (typical workload profile)

- Calibration parameters (tolerance, bounds)

2. {cpu}_cpu_model.py - Python implementation with:

- @dataclass QueueMetrics and CalibrationResult

- compute_stage_metrics(arrival_rate) method

- predict_ipc(arrival_rate) returning (ipc, metrics)

- calibrate(measured_ipc, tolerance) with binary search

- find_bottleneck(metrics) method

- Working example_usage() with test output

3. {CPU}_QUEUEING_MODEL.md - Full documentation (50+ pages) with:

- Architecture overview with historical context

- Queueing network diagram (ASCII art)

- Mathematical formulation with all equations

- Calibration protocol

- Validation results table

- References to datasheets and academic sources

4. QUICK_START.md - 5-minute getting started guide

5. README.md - Overview and file navigation

KEY MODELING REQUIREMENTS:

- Use M/M/1 queues for pipeline stages (M/M/1/K for finite buffers like caches)

- For modern CPUs: Add cache hierarchy stages (L1/L2/L3 with hit rates)

- For superscalar: Model multiple execution units as parallel queues

- For OoO: Add ROB modeling with instruction window effects

- For branch prediction: Model misprediction penalty as additional latency

- Target accuracy: <5% error after calibration

RESEARCH THESE SOURCES:

- Official processor datasheet and optimization manual

- WikiChip and CPU-World for microarchitecture details

- Agner Fog's instruction tables (for x86)

- Published benchmarks (SPEC CPU, Geekbench) for validation targets

MODERN CPU ADDITIONS (if applicable):

- SMT/hyperthreading: Model thread contention for shared resources

- Prefetch engines: Model hardware prefetch effectiveness

- Micro-op cache: Add front-end stage for decoded μops

- Turbo boost: Model frequency scaling effects

- Memory controller: Model DRAM latency and bandwidth

Please create all files with proper formatting, tested code, and thorough documentation consistent with the pre-1986 models in my repository.

=== END PROMPT ===

# 7. References

## 7.1 Academic Foundations

- Kleinrock, L. (1976). Queueing Systems, Volume II: Computer Applications

- Harchol-Balter, M. (2013). Performance Modeling and Design of Computer Systems

- Hennessy & Patterson (2017). Computer Architecture: A Quantitative Approach, 6th ed.

- Kennedy & O'Hagan (2001). Bayesian Calibration of Computer Models

## 7.2 Online Resources

- WikiChip: https://en.wikichip.org/

- CPU-World: https://www.cpu-world.com/

- Visual6502: http://www.visual6502.org/

- Agner Fog's Optimization Resources: https://www.agner.org/optimize/

- MAME Source Code: https://github.com/mamedev/mame

End of Document

Modeling_2026 Project | https://github.com/MartinGallagher-code/Modeling_2026